EXAMPLE:

I would like write 32256(decimal) on Pr118 and the drive has 3(decimal) of address with 19200bits/s

The serial protocol has this structure:

[STX]  [CMD + ADDR]  [BK + LUN]  [PAR]  [DO]  ...  [DN]  [CHK]

[STX]= [7E]

[CMD + ADDR]= see the table on the user manual, you find that CMD, for writing a parameter is 101(binary) and (for initial hipothesis) the ADDR is 3 (decimal).

So, [CMD + ADDR]= 101 + 00011, note that the bits must be eight --> [CMD + ADDR]=1010000(binary)=A3(hex)

[BK + LUN]= in the user manual you find: the BK field is represented by the 5 most significant bits and represents the 5 most significant bits of the parameter address. In this specific case you have Pr118 --> 118*2=236(decimal) = 11101100(binary) be carefull!! you must represent it on 13 bits and after consideronly th 5 significant bits.

11101100 (binary) --> 0000011101100 (binary) So the BK = 00000(binary).

LUN are 4 byte of the data trasmitted on 3 bits --> LUN=4(decimal)=100(binary)

Finally you have [BK + LUN]=00000 100 (binary)=02(hex)

[PAR] = represent the least significant eight bits of the address (Pr118 --> 118*2 = 236(decimal)= 11101100 (binary)= EC(hex))

[D0] ... [Dn] = in this example you have (Pr118=32256) 32256(decimal)=7E00(hex). Now take the digits in this manner:

(7E) (00) --> [D0]=00  [D1]=7E

If a field in the message different from STX assumes the value $7E, this field is followed by a 0 ($00) so that it will not be interpreted as an [STX].

At this point you can order all the (hex)data:

[STX]  [CMD + ADDR]  [BK + LUN]  [PAR]  [DO]  [D1]              [CHK]

[7E]           [A3]                   [02]          [EC]    [00]   [7E]   [00]     [CHK]

[CHK] = is the 256 module sum off all the field without [STX]

A3 + 02 + EC + 00 + 7E + 00 = 20F(hex) = 527(decimal)

527(decimal) / 256 =2

2 * 256 = 512 (decimal)

527(decimal) - 512(decimal) = 15(decimal) =F(hex)=[CHK] --> [CHK]=0F