Update Profinet Software:

Download ServoOne SW

- Start DM5

- Extras -> 011 Firmware loader -> Start firmware download

- Select file: 2000325aa.hex

- Wait 2-3 min, ServoOne displays L1 – L8, L3, L4, restart

- Ignore ServoOne error state

- Select “Don’ t perform initial commissioning

- Extras -> 011 Firmware loader -> Start firmware download

- Select file: 2000325aa.comwarc

Download Profinet SW

- Extras -> 011 Firmware loader -> Start firmware download

- Select file: Fieldbus/Profibus/PN0020099.comwarc

- Green LED at Profinet option is flashing, red is on

Behavior of LEDs at Profinet option:

Green is set:		Profinet option is ready

Green is flashing:	Blink function via Profinet or firmware is loading

Red is cleared: 	Drive is connected to PN Master

Red is set: 		Drive is not connected to PN Master

LED at Profinet connector:

Green is set: 		Bus is connected

Orange is set: 	Bus in state data exchange

Orange is flashing: 	Bus in state data exchange